CellProTM  Mycoplasma Scavenger Instructions

- Product Descriptions

| REF. | Product Name | Size |
| --- | --- | --- |
| CP-MS-0414 | CellProTM Mycoplasma Scavenger | 10μL |
| CP-MS-0414 | CellProTM Mycoplasma Scavenger | 100μL |
| CP-MS-0414 | CellProTM Mycoplasma Scavenger | 200μL |

Pre-use Instructions

Please keep the reagent tube in the instant centrifuge (3000 RPM, 3~5 s) after receiving the products.

Usage  Instructions

1.It is suggested that the separate to 10ul each storage and the products should be prepared before using.

2.When using Mycoplasma Scavenger, it is suggested that cells’ density should be maintained at 50 percent to 60 percent.

3.Recommended Mycoplasma Scavenger dilution ratio is 1:1000, for example that 10 mL medium added 10 Scavenger Mycoplasma Scavenger.

4.Removed the old medium, cleaned the cells with PBS, and added a fresh medium containing Mycoplasma Scavenger, 1 times one day, 3 days for continuous treatment, or 5~6 days for 5~6 days 1 times for 2 days. If cell contamination is very serious, it is necessary to extend the treatment time.

5.If cells are sensitive to Treatment or growth is obviously inhibited, please refer to the following recommended parameters to process cells.

CellProTM  Mycoplasma Scavenger

| Cells Type | Normal Cells | Sensitive Cells | Extremely Sensitive |
| --- | --- | --- | --- |
| Dilution Ratio | 1/1000 | 1/2000 | 1/3000 |
| Treatment Time | 3 Days | 6 Days | 10 Days |

- Added the fresh medium when the treatment was over.

- Storage Instructions

This product （CP-MS-0414)） kept under 4 ℃ for 6 months or kept under -20 ℃ for 12 months .Upon delivery aliquot,Avoid freeze / thaw cycle.

- Supplementary

- Because of Mycoplasma Scavenger itself also have broad spectrum antimicrobial resistance, in order to reduce the influence on cell, it is strongly recommended that don't use and other antibiotics at the same time.

- Mycoplasma Scavenger has been tested in over 100 cell lines, including but not limited to CACO2,HCT116,A549,LINcap,AGS,BGC823,HepG2, HEK293, Hela, MCF-7, NIH3T3, , H295R, HL60, K562, , SP20, T47D, HT29,HEY etc..